MONTANA WELL LOG REPORT; Other Options. This well log reports the activities of a licensed Montana well driller, serves as the official record of work done within the borehole and casing, and describes the amount of water encountered. This report is compiled electronically from the contents of the Ground Water Information Center (GWIC) database for this site. Acquiring water rights is the well owner's responsibility and is NOT accomplished by the filing of this report.; Return to menu Plot this site on a topographic map. Site Name: BREKKE, RAY & KARI GWIC Id: 228799. Section 1: Well. 5925 LINCOLN RD. HELENA MT 59602 [05/05/2006]. Section 2: Location. Township; Range; Section; Quarter Sections. 11N; 04W; 6; NW¼ NW¼. County; Geocode. LEWIS AND CLARK. Latitude; Longitude; Geomethod; Datum. 46.746345; 112.166878; TRS-SEC; NAD83. Ground Surface Altitude; Method; Datum; Date. Addition; Block; Lot. MANLEY O MARTENSON; 2. Section 3: Proposed Use of Water. DOMESTIC (1). Section 4: Type of Work. Drilling Method: ROTARY. Section 5: Well Completion Date. Date well completed: Friday, May 05, 2006. Section 6: Well Construction Details. Borehole dimensions From; To; Diameter. 0; 180; 6.. Casing From; To; Diameter; Wall Thickness; Pressure Rating; Joint; Type. 2; 18; 6; 0.250; WELDED; A53B STEEL. 10; 180; 4; 200.00; WELDED; PVC-SDR 200.. Completion (Perf/Screen) From; To; Diameter; # of Openings; Size of Openings; Description. 80; 120; 4; .020; FACTORY SLOTTED. 160; 180; 4; .020; FACTORY SLOTTED.. Annular Space (Seal/Grout/Packer) From; To; Description; Cont. Fed?. 0; 18; BENTONITE; Y..; Section 7: Well Test Data. Total Depth: 180 Static Water Level: 70 Water Temperature: Air Test * 15 gpm with drill stem set at 180 feet for 1 hours. Time of recovery 0.18 hours. Recovery water level 70 feet. Pumping water level feet. * During the well test the discharge rate shall be as uniform as possible. This rate may or may not be the sustainable yield of the well. Sustainable yield does not include the reservoir of the well casing.. Section 8: Remarks. Section 9: Well Log. Geologic Source. Unassigned. From; To; Description. 0; 70; CLAY. 70; 180; HARD CLAY.. Driller Certification. All work performed and reported in this well log is in compliance with the Montana well construction standards. This report is true to the best of my knowledge.. Name:. Company:; H & L DRILLING INC. License No:; WWC-447. Date Completed:; 5/5/2006...