# FIELD NOTES FOR A BOUNDARY SURVEY

## COMPLETED NOVEMBER 20, 2010

(17.98 ACRES)

17.98 acres of land, lying in Dimmit County, Texas being out of and a part of Farm Tracts numbered 30, 31, 32, 33, 34, and 35 in Section No. 172 of the Cross S Ranch Subdivision according to the part thereof recorded in Volume 1, Page 11 of the Plat Records of Dimmit County, Texas and being that same certain 18.00 acres described in conveyance document to Anabel Ramirez, et vir and recorded in Volume 302, Pages 737-742 of the Official Public Records of Dimmit County, Texas and said 17.98 acres being more particularly described by metes and bounds as follows: (The bearings and distances shown herein conform to the Texas Coordinate System, North American Datum 1927, Texas South Central Zone.)  (All corners called for as being set are marked on the ground with ½” diameter steel stakes with identification markers stamped “SMYTH/2046” attached unless otherwise noted or shown.):

BEGINNING at a steel stake with an aluminum cap marked “Wolff Surveys 1502” found near 3-way fence corner for the south corner of the herein described tract and from which the ostensible southeast corner of the E. Campbell Survey No. 269 1/2, Abstract No. 943 bears N 19° 13’ 52” W at a distance of 2922.04 feet;

THENCE: N 19° 13’ 52” W with the common line of the herein described tract and that same certain tract of land described in conveyance document to William Campbell Davisson, et al and recorded in Volume 198, Pages 558-560 of the Deed Records of Dimmit County, Texas and generally with occupied fence, at 362.98 feet cross the ostensible northwest line of said Farm Tract 33, at 816.22 pass a  ¾” steel stake found under fence and on line, continuing for a total distance of 868.05 feet to a ¾” steel stake with aluminum cap marked “Wolfe Surveys 1502” found near 3-way fence corner for the west occupied corner of the herein described tract;

THENCE: N 70° 49’ 39” E, with a common line of the herein described tract and said Davisson tract and generally with occupied fence, at 55.80 feet cross the ostensible southwest line of said Farm Tract 31, at 715.30 feet cross the ostensible northeast line of said Farm Tract 31, at 793.78 feet pass a ¾” steel stake with aluminum cap marked “Wolff Surveys 1502” found under fence and on line, continuing for a total distance of 1006.89 feet to a 3” iron pipe 3-way fence corner found for north corner of the herein described tract and being at a point on the northwest right-of-way line of U.S. Highway No. 83 (150 ft. right-of-way);

THENCE: S 05° 33’ 24” E, with said northwest right-of-way line of U.S. Highway No. 83, at 420.00 feet pass a ¾” steel stake with aluminum cap marked “Wolff Surveys 1502” found under fence, at 518.74 feet pass the ostensible northwest line of said Farm Tract 35, continuing for a total distance of 771.10 feet to a TXDOT concrete monument for a deflection point;

THENCE: S 11° 18’ 34” E, with said right-of-way line of U.S. Highway No. 83 and generally with occupied fence for a distance of 119.67 feet to a 3/4” steel stake found about 1 ft. below the surface of asphalt for the southeast entrance gate corner of the herein described tract;

THENCE: S 70° 49’ 20” W, with the common line of the herein described tract and said Davisson tract, at 93.15 feet cross the ostensible northeast line of said Farm Tract 34, at 753.30 feet pass a ¾” steel stake found at the southwest line of said Farm Tract 34, continuing for a total of 808.10 feet to the Place of Beginning and containing 17.98 acres of land within the herein described tract as surveyed by D. G. Smyth & Co., Inc. on November 20, 2010.

THE STATE OF TEXAS:

COUNTY OF UVALDE:

It is hereby certified that the foregoing field note description and

Attached plat were prepared from an actual on the ground survey

Made by personnel working under my direct supervision and that

Same are true and correct according to same said survey.

____________________________________________________

D. G. Smyth-Registered Professional Land Surveyor/

Licensed State Land Surveyor-No. 2046

### JOB # 10-4082