Land in Avinger, TX

Being all that certain 38.961 acre tract of land situated in the Jonathan Hopkins Survey, A-210 in Marion County, Texas and being the residue of the called 220 acre tract conveyed from Thomas Nelson, Trustee, to the Lone Star Iron Company by deed dated January 1st, 1891 and recorded in Volume W on Page 218 of the Deed records of Marion County, Texas, said 38.961 acre tract of land being more particularly described as follows:

Beginning at the ¾” iron rod found for an ell corner of the Robert B. Fowler Survey A-133 and the Northeast corner of the said Jonathan Hopkins Survey, same being the Northeast corner of this tract, from which a 16” Blackjack bears S 08°E, 22.0 ft. and a 23” hickory bears S 76° W, 47.5 ft., both marked with an old “X”;

THENCE: S 00° 53’ 09” W, with the Southerly West line of the said Fowler Survey and the East line of the said Hopkins Survey, at 556.20 ft. cross the center of a graded County Road, and continuing S 00° 53’ 09” W, with said lines for a total distance of 2546.30 ft. to a 3/4” Iron Rod found for the Southeast corner of this tract and the most Easterly Northeast corner of the called 192 acre United States of America Tract No. 11-809, from which a 14” Red Oak marked “X” bears S 08° 30’ W, 33.8 ft. and a 19” White Oak marked with an old “X” bears S 64° W, 17.5 ft.;

THENCE:  N 36° 47’ 06” W, with the U.S.A. fee line 614.72 ft. to a concrete monument with a bronze disk marked 11-29 for the most Southerly Southwest corner of this tract, from which a 6” Holly bears N 12° E, 8.7 ft. and a 25” Blackjack bears S 22° E, 16.2 ft., both marked with an old “X”;

THENCE:  N 00° 16’ 09” E, continuing with the U.S.A. fee line, 1251.19 ft. to a concrete monument with a bronze disk marked 11-23 for an ell corner of this tract, from which a 15” Post Oak bears N 35° E, 13.5 ft. and a 17” Red Oak bears S 49° W, 11.2 ft., both marked with an old “X”;

THENCE:  S 68° 15’ 49” W, continuing with the U.S.A. fee line, 666.64 ft to a concrete monument with a bronze disk market 11-27 an intermediate Southwest corner of this tract, from which an 11” Sweetgum bears N 77° E, 11.0 ft. and a 12” Sweetgum bears N 64° W, 22.6 ft., both marked “X”;

THENCE:  N 21° 40’ 16” W, continuing with the U.S.A. fee line, 1154.14 ft. to a concrete monument with a bronze disk marked 11-26 in the North line of the said Hopkins Survey for the Northwest corner of this tract and the most Northerly Northeast corner of the said Tract No.  11-809, from which a 24” Pin Oak marked with an old “X” bears S 87° E, 30.0 ft.;