LEGAL DESCRIPTION

BEING a tract of land out of the T.J. Tone Survey, Abstract No. 829, Hamilton County, Texas and being part of a called 43.86 acre tract of land described in deed to Carolyn Williams and husband, Ronald H. Williams recorded in Volume 372, Page 171 of the Real Property Records, Hamilton County, Texas and being more particularly described as follows:

BEGINNING at a 3/8” iron rod found in the west line of said T.J. Tone Survey for the southwest corner of said 43.86 acre tract and being the southwest corner of this tract; (Grid Coordinates: N:10574417.86, E:2917202.94)

THENCE with said west line of said T.J. Tone Survey, North 17°13'12" East, a distance of 2090.92 feet to a point for the northwest corner of this tract from which a 3” pipe corner post found for the northwest corner of said 43.86 acre tract bears North 17°13'12" East, 12.11 feet;

THENCE departing said west line, over and across said 43.86 acre tract with a fence, South 73°17'26" East, a distance of 900.04 feet to a 3” pipe corner post found in the east line of said 43.86 acre tract for the northeast corner of this tract from which a 3” pipe corner post found for the northeast corner of said 43.86 acre tract bears North 17°13'11" East, 324.13 feet;

THENCE with said east line of said 43.86 acre tract, South 17°13'11" West, a distance of 2095.33 feet to a 1/2" iron rod with plastic cap stamped “H&P RPLS 6827” set for the southeast corner of said 43.86 acre tract and being the southeast corner of this tract;

THENCE with the south line of said 43.86 acre tract, North 73°00'35" West, a distance of 900.02 feet to the POINT OF BEGINNING and containing 43.2466 acre or 1,883,823 square feet of land.

Bearing system based on the State Plane Coordinate System, Texas Central Zone (4203), North American Datum of 1983 (NAD83) per the AllTerra Trimble Global Positioning System RTKnet.